Image analysis- calculating the MDM survival ratio based on total intensity of CellTracker in segmented objects per well

Output from Hermes

- Folder for each image acquisition with date and time e.g. CT081_IA_050523_1614

  - Subfolders

    - Images: contains a folder for each well. Within these .tif files of each channel imaged (blue/ green)

      - Each .tif file contains 51 fields of view

    - CT081_IA_050523_1614/Reports

      - Contains html file that gives details of image acquisition e.g. focus/ exposure

Image pre-processing (ImageJ)

- Within your main folder for each acquisition make two new subfolders

  - Separated_images_blue

  - Separated_images_green

- Open ImageJ

  - Navigate to Plugins/Macros/Interactive Interpreter

  - Copy and paste the code from the file “Stack_to_separate_images_all_subfolders_CT_14.11.21” into the interactive interpreter

    - Update the dir in “run("Image Sequence... ", "dir=[/Volumes/Backup/CT082/CT082_96h_IA_150523_1515/Separated_blue] format=TIFF name=["+name+"] start=1 digits=3");” to path for the “Separated_images_blue” you have just made for this experiment

      - In finder: right click while holding down “option” allows you to copy path name

  - In the macros tab select “Run macro”

    - A finder window will appear. Navigate to the “Images” subfolder within your experiment. Click on it so it is highlighted in blue then select “Open”- this will start the macro running

Moving separated images

- All the separated images (both blue and green channel) will end up in the “Separated_images_blue” folder

- To move the green channel images to your Separated_images_green folder copy and paste the following code into terminal

  - find x -iname '*Green*' | xargs -I '{}' mv '{}' y

    - Replace x with the path name of the “Separated_images_blue” folder

    - Replace y with the path name of the “Separated_images_green” folder

  - Check the folders to ensure images have moved as expected (total number in each should be 51x number of wells imaged)

- If you multiplexing several plates for analysis rename all the images within the “Separated_images_green” with a prefix for the plate

  - Select all images- right click – rename- select “Add text” option- add a prefix e.g. Expt_plate_

CellProfiler pipeline

- Make a small sub-selection of test images for experiment

  - Try to select images that cover the breadth of acquired images e.g. some with no signal, some with lots, some with a mixture of bright and dark signal

- Open CellProfiler and load project file by dragging it onto left hand side of CellProfiler GUI

  - You will get a prompt asking if you want to load the project- click “Yes”

  - Pipeline modules appear down left side of window

- Prep

  - Re-save the project under a specific name for this experiment

  - Click on the “Images” tab and clear the Input image list

    - If there are images listed: select all, right click and select “Remove from list”

  - In the “Export to spreadsheet” tab

    - Check that the “Output file location” is correct

    - Update the “Filename prefix” to something unique for the experiment

      - Separate words with underscore (_)

- Test pipeline

  - Add test images for this experiment

    - Click on the “images” tab and lick and drag all images from your test images folder onto the central panel. They should appear in a list

  - Go to “Identify primary objects” tab

    - These are the parameters that determine what CellProfiler detects as an object

    - Two parameters can be altered

      - Threshold correct factor: lower values are more lenient and objects bigger. Higher values more stringent and objects smaller.

        - Tend to use values between 0.65 and 0.75

      - Lower and upper bounds on threshold

        - Leave upper bound at 1.0

        - Lower bound typically 0.04

          - Signal has to be more than this for CellProfiler to measure it as real signal

          - Sometimes need to edit this a little e.g. 0.03-0.05 depending on background signal/noise in image acquisition

    - Test pipeline (only need to test first stage i.e. identification of primary objects”)

      - Click “Start Test mode”

      - Click “Step”. A window with object segmentation for first image will appear

        - To move to next image, click “Next Image Set” and then “Step”

        - Repeat this for all images in test set

      - You can modify the Threshold correct factor and Lower bound on threshold while in test mode and see effect on object segmentation

    - Once you are happy with parameters click “Exit test mode”

    - Save the project

- Running the pipeline

  - Clear the test images from Input image list

    - Go to the “Images” tab and clear test images from Input list

      - Select all, right click and select “Remove from list”

  - Add your images for analysis to the input list

    - Click on the “Images” tab and lick and drag all images from your “Separated_images_green” onto the central panel. They should appear in a list

  - Click “Analyse Images”

    - A warning will appear asking if you are sure you want to use a spreadsheet for a large number of images- say yes

  - The pipeline will run

    - Suggest setting up overnight- it’s CPU intensive and difficult to use laptop at same time

  - A notification will appear at the end to say it’s complete

Calculating survival ratio

- Output from CellProfiler

  - Three .csv files

    - _Image.csv

      - Has summary measurement for each FoV- this is what we use for analysis

    - _objects.csv

      - Every single CellTracker object

    - _Experiment.csv

      - Analysis parameters (meta data)

- Suggest making an excel sheet (or R) to summarise results (see example)

  - In excel I have one tab to calculating the total intensity of CellTracker for every well in alpha-numeric order and one tab that organises the wells by experimental condition (easier for analysis)

- Calculating total intensity of CellTracker within segmented objects in a well

  - Open the _image.csv output file

  - The column with the relevant data is “Intensity_TotalIntensity_CellTracker_CellTracker_objects” – normally column Q

    - Each row is a FoV- 51 for each well

  - In your analysis spreadsheet sum the 51 FoVs for each well to give a total.

    - If you have only one plate per experiment can use “SUMIF”

      - =SUMIF(CellTracker_CT082_Image.csv!$E:$E,"*"&D2&"*",CellTracker_CT082_Image.csv!$Q:$Q)

        - Sums data in column Q based on data in column E (FileName)

          - "*"&D2&"*" allows you to look up well IDs if you have a list of these

    - If you have multiplexed multiple plates use “SUMIFS”

      - =SUMIFS(CellTracker_CT082_Image.csv!$Q:$Q,CellTracker_CT082_Image.csv!$E:$E,"*"&D2&"*",CellTracker_CT082_Image.csv!$E:$E,"*CT082_24h*")

        - Sums data in column Q based on data in column E (FileName)

          - "*"&D2&"*" allows you to look up well IDs if you have a list of these

          - "*CT082_24h*" allows you to look up plate prefix

- Calculate survival ratio for each well

  - Divide the value total intensity for each well by the mean of values from your 3 control wells (MDM + media)